/*!

\page warping_page Warping

\image html image24.gif

\n \b Warping indicates that a face is not planar and is applied only to

2D elements with 4 nodes. This quality control criterion is based on a

projection plane created by:

<ol>

<li>bisecting the four element edges,</li>

<li>creating a point on the plane at the vector average of the

corners, where the x-axis extends from the point to the bisector on

edge 2.</li>

</ol>

The plane normal is in the direction of the cross product of the

x-axis and the vector from the origin to the bisector of edge 3. Every

corner of the quad will then be a distance ?h? from the plane. The

length of each half edge is measured and the shortest length is

assigned ?l?. The warp angle is the arcsine of the ratio of the

projection height ?h? to the half edge length ?l?.

<em>To apply the Warping quality criterion to your mesh:</em>

<ol>

<li>Display your mesh in the viewer.</li>

<li>Choose <b>Controls > Warping Angle</b> or click <em>"Warping angle"</em>

button of the toolbar.

\image html image39.png

<center><em>"Warping angle" button</em></center>

Your mesh will be displayed in the viewer with its elements colored

according to the applied mesh quality control criterion:

\image html image97.jpg

</li>

</ol>

<br><b>See Also</b> a sample TUI Script of a

\ref tui_warping "Warping quality control" operation.

*/